Parts

- Ardunio Mega

- Dual G2 High-Power Motor Driver 24v14  Shield

- Power supply box

- Breadboard / circuit board

- 10 K resistor

- 12 Wires

- Bucket

- Fountain Pump 150

- 2 water tubes

- Rectangular module with center hole Peltier (max volts 6.1)

- Thermistor

- Water block

- Sapphire window

Hardware setup

Arduino Mega diagram:

- Dual G2 High-Power Motor Driver 24v14 Shield diagram

Solder the shield pins to the shield then place it on top of the arduino.

M1A/M1B are used for the motor ( arduino shield ) (*kind of arduino shield)

M2A/M2B can be used for another arduino setup or another motor

GND/VIN connects to the power supply

Use the Library Manager to install this shield library:

- In the Arduino IDE, open the "Sketch" menu, select "Include Library", then "Manage Libraries...".

- Search for "DualG2HighPowerMotorShield".

- Click the DualG2HighPowerMotorShield entry in the list.

- Click "Install".

Attaching the thermistor to the Peltier

- Use a good amount of thermal paste to the peltier and attach the Sapphire widow, make sure you push down hard to create a thin layer of thermal paste

- Clean up thermal paste around window with isopropyl alcohol

- Add super glue if needed

- Make sure widow doesn’t slide

- Attach thermistor near the sapphire window with super glue, also use copper tape to assist with the hold.

Attaching the waterblock to the Peltier

- Apply thermal paste to waterblock and attach the peltier to it and apply pressure.

Waterblock

Use any bucket with the Fountain Pump 150

Software setup

Tools / board / “ Arduino Mega”

Tools / processor / “ Arduino Mega”

Tools / port / “ Arduino Mega”

Code:

#include "DualG2HighPowerMotorShield.h"

//Thermometer with thermistor

/*thermistor parameters:

* RT0: 10 000 Ω

* B: 3977 K +- 0.75%

* T0:  25 C

* +- 5%

*/

//These values are in the datasheet

#define RT0 10000   // Ω

#define B 3977      // K

//-----------------------------------------------------------

#define VCC 5    //Supply voltage

#define R 10000  //R=10KΩ

//Variables

float RT, VR, ln, TX, T0, VRT;

double Setpoint;//

unsigned long time; //

int x = 40 ; // can change this number to whatever// basically the second setpoint

int x2 = 45; //  3rd setpoint

int x3= 50;   // 4th setpoint etc // can add as many increments or decrements as needed

DualG2HighPowerMotorShield24v14 md;

void stopIfFault() //failsafe function to disable the M1 Output if there is a "fault"

{

if (md.getM1Fault())

{

md.disableDrivers();

delay(1);

Serial.println("M1 fault");

while (1);

}

}

void setup()

{

Serial.begin(115200);

Serial.println("Dual G2 High Power Motor Shield");

md.init();

md.calibrateCurrentOffsets();

delay(10);

Serial.begin(9600);

T0 = 25 + 273.15;                 //Temperature T0 from datasheet, conversion from Celsius to kelvin

}

// -------------------------------------------------------------------------------------------

void loop()

{

//Thermistor reading and printout

VRT = analogRead(A7);              //Acquisition analog value of VRT

VRT = (5.00 / 1023.00) * VRT;      //Conversion to voltage

VR = VCC - VRT;

RT = VRT / (VR / R);               //Resistance of RT

ln = log(RT / RT0);

TX = (1 / ((ln / B) + (1 / T0))); //Temperature from thermistor

TX = TX - 273.15;                 //Conversion to Celsius

//---------------------------------------------------------------------------------------------------------------------------

//PID equation with thermistor as input

/*working variables*/

unsigned long lastTime;

double Input, Output;

double errSum, lastErr;

double kp=45, ki=0, kd=0;

/*How long since we last calculated*/

unsigned long now = millis();

double timeChange = (double)(now - lastTime);

/*Compute all the working error variables*/

Input = TX; //using the thermistor temp in deg Celsius as the PID input

Setpoint = 35; // original setpoint// The goal temperature of the PID in deg Celsius

if ( TX <  70 ) // leave this number at 70 unless your setpoint is above 70 then increase it             accordingly

{

Setpoint = Setpoint + 0.63

; // add the offset value of the setpoint

}

//setpoint          offset value

//25                  0.13

//26                  0.17

//27                  0.2

//28                  0.31

//29                  0.32

//30                  0.32

//31                  0.41

//32                  0.48

//33                  0.45

//34                  0.6

//35                  0.63

//36                  0.66

//37                  0.77

//38                  0.86

//39                  0.84

//40                  0.91

//41                  0.95

//42                  0.98

//43                  1

//44                  1.1

//45                  1.08

//46                  1.14

//47                  1.19

//48                  1.22

//49                  1.23

//50                  1.34

//51                  1.43

//52                  1.5

//53                  1.55

//54                  1.72

//55                  1.72

// 1 min = 60000

// 5 mins = 300000

// 10 mins = 600000

// 30 mins = 1800000

// 1 hr = 3600000

// 2 hr = 7200000

if ( now  > 1800000)  // if time is Greater then ____ mins //

//if sepoint>x

{ Setpoint = Setpoint + (x-Setpoint);       // when condition is true, setpoint is added to the x value to become the new setpoint //

// When you want to do decrement use this equation, Setpoint = Setpoint + (Setpoint-x);

}

if ( now >3600000) //

{ Setpoint = Setpoint + (x2-Setpoint);  // second increment/ decrement

}

if ( now >5400000) // *updated // third increment/ decrement

{ Setpoint = Setpoint + (x3-Setpoint);

}

double error = Setpoint - Input;

errSum += (error * timeChange); //integral-type calculation of error vs. time

double dErr = (error - lastErr) / timeChange;  //derivative-type calculation of change in error since last calculation

/*Compute PID Output*/

Output = kp * error + ki * errSum + kd * dErr; //THE PID EQUATION

/*Remember some variables for next time*/

lastErr = error;

lastTime = now;

//----------------------------------------------------------------------------------------------------------

//Print the peltier Output value (unknown units), and the thermistor temp (deg Celcius)

Serial.print("Output:");          //Print the PID response

Serial.print(Output);

Serial.print("\t");

Serial.print("Temperature:");     //print the temperature

Serial.print("\t");

//Serial.print("F ");

//Serial.print((TX * 1.8) + 32);    //Conversion to Fahrenheit

//Serial.print("\t\t");

//Serial.print("K ");

//Serial.print(TX + 273.15);        //Conversion to Kelvin

Serial.print("\t\t");

//Serial.print("C ");

Serial.println(TX);

delay(250);

//--------------------------------------------------------------------------------------------------

md.enableDrivers();

delay(1);  // The drivers require a maximum of 1ms to elapse when brought out of sleep mode.

if ( TX > 75) // If temp goes above the set value then the program stops running

{

md.disableDrivers();

delay(1);

}

if ( Output > 0) //

{

{md.setM1Speed(Output);

stopIfFault();

}

{

Serial.print("M1 current: ");

Serial.println(md.getM1CurrentMilliamps());

}

delay(2);

}

if (Output <= 0)

{

//md.disableDrivers();

md.setM1Speed(Output);

stopIfFault();; // Put the MOSFET drivers into sleep mode.

delay(200);

}

}

Troubleshooting

I get an uploading error “ selected port avrdude”?

Make sure the Arduino is plugged into the computer

Make sure under  Tools / port  that you see “ Arduino Mega”

Check to see if all the wires are connected the right way via the schematic

I see the green light on the arduino that signifies that the arduino is on but my computer doesn’t recognize it in the port ?

Make sure that all the wires are soldered properly.

I have the pid programmed to increase in temperature but instead it decreases in temperature?

Switch the black and red wire on the arduino shield ( M1A/M1B )